EXPERIMENT 10

CONTINUOUS-TIME LTI SYSTEM RESPONSE ANALYSIS

OBJECTIVE:

To distinguish the responses produced by initial stored energy and external excitation, obtain the impulse response of a continuous-time LTI system, and verify that the total response is the sum of the zero-input and zero-state responses, with the zero-state response also obtained through convolution.

CODE :

clc;

clear;

close all;

% System: dy(t)/dt + 2y(t) = x(t)

% Transfer function H(s) = 1/(s+2)

num = [1];

den = [1 2];

sys = tf(num, den);

t = 0:0.01:5;

dt = t(2) - t(1);

% Impulse response

h = exp(-2*t);

figure;

plot(t, h, 'LineWidth', 1.5);

grid on;

title('Impulse Response');

xlabel('Time (s)');

ylabel('h(t)');

% Zero-input response: y(0-) = 1

y_zi = exp(-2*t);

figure;

plot(t, y_zi, 'LineWidth', 1.5);

grid on;

title('Zero-Input Response');

xlabel('Time (s)');

ylabel('y_{zi}(t)');

% Unit-step input

x = ones(size(t));

% Zero-state response using convolution

y_conv = conv(x, h) * dt;

y_zs_conv = y_conv(1:length(t));

% Zero-state response using lsim

y_zs = lsim(sys, x, t);

figure;

plot(t, y_zs, 'LineWidth', 1.5);

hold on;

plot(t, y_zs_conv, '--', 'LineWidth', 1.5);

grid on;

title('Zero-State Response');

xlabel('Time (s)');

ylabel('y_{zs}(t)');

legend('Using lsim', 'Using Convolution');

% Total response

y_total = y_zi + y_zs;

figure;

plot(t, y_total, 'LineWidth', 1.5);

grid on;

title('Total Response');

xlabel('Time (s)');

ylabel('y(t)');

disp('Transfer function:');

sys

disp('Pole of the system:');

pole(sys);