Experiment 12

Causality and Non-Causality

Objective: To distinguish between causal and non-causal systems using MATLAB.

CODE :

% Experiment 12: Causality and Non-Causality

clc;

clear;

close all;

n = -10:10;

x = sin(0.2*pi*n);

% Causal system: y[n] = x[n] + x[n-1]

yC = x + [0 x(1:end-1)];

% Non-causal system: y[n] = x[n] + x[n+1]

yNC = x + [x(2:end) 0];

figure;

subplot(3,1,1);

stem(n,x,'filled');

grid on;

title('Input x[n]');

xlabel('n');

ylabel('Amplitude');

subplot(3,1,2);

stem(n,yC,'filled');

grid on;

title('Causal System y[n] = x[n] + x[n-1]');

xlabel('n');

ylabel('Amplitude');

subplot(3,1,3);

stem(n,yNC,'filled');

grid on;

title('Non-Causal System y[n] = x[n] + x[n+1]');

xlabel('n');

ylabel('Amplitude');